Chad Perron

266 Willow Brook Road

469-3820

272-2420

Permit # 2019-47

Map/Lot 225-03

porch and generator and garage

06-27-19	Chad called to say he had some drawings for his porch and also wanted to have a generator installed and wanted to know what I required for information and what inspections were required. He said the generator would be installed later this summer and I told him to put both on permit.

Time 0.2

10-11-19	Larry Davis said garage forms would be ready today via email.

Went by trench dug but no forms. Power to house routed around trench as conduit was removed from pedestal to other side of trench.

Time 0.7

10-11-19	Went by in afternoon as she said they were coming this morning. Trench was made wider and smoother bottom.

Time 0.2

10-14-19	Went by to see if they were ready, no activity.

Time 0.3

10-16-19	Got a call to inspect the footer forms, looked okay, rebar supported on chairs, tied together, ground wire ran from house to where pedestal is as they are not sure where the meter panel will be yet.

Time 0.5

10-18-19	Inspected the wall forms, plenty of rebar, will not have basement under addition.

Time 0.2

11-04-19	Chad called and wanted me to inspect the forms for the piers for the three sided porch. Looked okay and emailed him it was okay to pour.

Time 0.6

(Date?) 266 Willow Brook Rd: Perron – Electrical- pre-meter relocate:

Met owner’s builder (Larry Davis) on-site for pre-service move inspection. Previous entrance was on side of house. Will be moved to corner of new attached garage. Walked through project (new attached garage and wrap-around porch.) New panel location on eave side of building has been approved by utility co. per builder. New garage is closed in tight to weather except for garage doors. Everything looked ok for new service installation, but nothing was installed yet. I indicated that I’d be happy to return when the electrician was further along, but utility co specs the entrance criteria, we inspect from there in.

From: "Larry Davis" <larry@dabvt.com>
To: <inspector@plainfieldnh.org>
Subject: request for inspection
Date: Tue, 8 Oct 2019 20:41:03 -0400

THIS IS IN REGARDS TO PERMIT # 2019-47

CHAD & VANESSA PERRON

266 WILLOW BROOK RD.

Currently hole for the garage foundation has been dug.

Foundation contractor anticipates setting the footing boards Thursday and pouring them Friday Afternoon.

Would you please inspect the footings as you require Friday AM . if there is a problem please call my cell and let me know.

Cell # 802 280 5586

Thank you.Dave

Majority of framing is complete at 266 Willow Brook Rd.

Majority of electrical is rough in waiting on utility CO. to put Meter panel back on the new garage.

Was hoping to get on your schedule sometime soon.

Please let me know when you can come for an inspection.

Thank you.

Larry Davis,

From: David Lersch

Sent: Wednesday, October 9, 2019 10:23 AM

To: larry@dabvt.com

Subject: Re: request for inspection

EIther have contractor call me or let him know that rebar must be supported, all rebar tied together, 20' of copper wire clamped to rebar at end and where panel is located, plus enough to reach your panel (leave a coil)

---------- Original Message ----------
From: "Larry Davis" <larry@dabvt.com>
To: <inspector@plainfieldnh.org>
Subject: request for inspection
Date: Tue, 8 Oct 2019 20:41:03 -0400

THIS IS IN REGARDS TO PERMIT # 2019-47

CHAD & VANESSA PERRON

266 WILLOW BROOK RD.

Currently hole for the garage foundation has been dug.

Foundation contractor anticipates setting the footing boards Thursday and pouring them Friday Afternoon.

Would you please inspect the footings as you require Friday AM . if there is a problem please call my cell and let me know.

Cell # 802 280 5586

Thank you.

Larry Davis,

David

currently foundation is poured & backfilled. Footing & tubes are in waiting for concrete. Re-bar to be installed during pour. All footing tubes have a base ( big foot on them) and are backfilled with crushed stone up about 2”"’ on the tube.

If you would like to inspect I would appreciate that.

Thank you.

Larry Davis,
Davis Alterations & Building, LLC
1235 Harlow RD.
South Royalton, Vermont 05068
802 763 8666
larry@dabvt.com
www.dabvt.com

From: "Larry Davis" <larry@dabvt.com>

To: <inspector@plainfieldnh.org>

Subject: Inspection

Date: Mon, 16 Dec 2019 19:33:34 -0500

Dave Majority of framing is complete at 266 Willow Brook Rd. Majority of

electrical is rough in waiting on utility CO. to put Meter

panel back on the new garage. Was hoping to get on your schedule sometime

soon. Please let me know when you can come for an inspection. Thank you.

Larry

Davis,

David H Lersch

Plainfield Building Inspector