Michael O’Leary

80 Ladieu Road

298-0535

Permit # 5029

Permit # 7064

Map/Lot 234/18

28’x42’ SFH

05-06-05	Permit 5029, 80 Ladieu Road. Talked to Michael about his SFH and what inspections were necessary.

Time 0:10

05-26-05	Permit 5029, 80 Ladieu Road. Inspected footer and  concrete walls that were poured already and forms for back wall. Conduit for electrical in place and main plumbing pipe run from inside foundation to septic tank. Buried electrical cables and conduit already installed and covered from the pole down at the end of the driveway to the meter approximately 2/3 up the driveway. From the meter to the foundation just the conduit has been installed and buried. From what I could see everything appeared okay.

Time 0:25

10-04-05	Permit 5029, 80 Ladieu Road. Inspected plumbing in basement so that the floor can be poured. Discussed construction of outside decks and progress so far. Roof being shingled, house will have radiant heat, and placement of the boiler and venting.

Time 0:35

03-10-06	Permit 5029, 80 Ladieu Road. I met with Michael to do the final rough in inspection as they were getting ready to install the sheetrock. The two story home has radiant heat for the first floor and hot water baseboard for the second floor. The insulation was blown in cellulose and it looks like it should be a very well insulated house. I told Michael to go ahead and close in the walls and to call me when the painting is the only thing that needs to be done.

Time 0:45

12-02-20	Met with Mike to perform a final inspection on a long expired permit as they want to sell the house, letter sent in 2010 stating permit expired in 2008. Second floor railing is not all finished yet, A few locations need a smoke or smoke/co detector, discharge pipe on unit for radiant and baseboard hot water heating needs installed. Codes at the time were 2006 IRC and 2003 Life Safety Codes. Could not verify if penetrations were fire stopped as all ceilings were finished. Garage as well as unfinished garage second floor need to have 1/2 inch sheetrock installed on walls that are common with living space. Mike will work on those items and I will draft a letter stating that from what I could see it should meet the codes at that time, no Certificate of Occupancy will be given.

Time 1.4